## 1. Logic Unit

"The core of a processor is the ability to select different operations based on input. We start with a unit which select between four different bitwise-logic operations."

op1 and op0 determine operations performed on X and Y

Halved by SELECT from op0 then determined by SELECT from op1

## 2. Arithmetic Unit

Same 4 operation selection performed with 3 SELECT gates,

  - Increment done by adding inverse of zero

  - Decrement done by adding inverse of zero of all 16 bits (negative sign plus reversed 1's and 0's, representing a negative 1)

  - Add done with simple add operation

  - Subtract done with simple subtract operation

## 3. Arithmetic Logic Unit

When the sw flag is 1, the X and Y inputs are swapped.

When the zx flag is 1, the left operand is replaced with 0.

  - Condition

Solution using ANDs

More efficient solution using NANDs