## 1. Combined Memory

"A processor uses both kinds of memory, registers, and RAM."

"Registers are directly accessible by the processor and used for intermediate values and calculations. RAM can store a large amount of data, but we can read or write from only a single address at a time."

"In this processor we have two registers called A and D and one RAM bank."

## 2. Instruction

"I is an instruction to the ALU and condition components. The bits direct the operation as specified:"

"The A, D and *A inputs are the values of the registers"

"The X input to the ALU should be D, the Y input should be either A or *A depending on bit 12 in the instruction. If bit 12 is 0, it is A, if 1, *A."

"The R output is the result of the ALU operation."

"The j flag indicate if the ALU output conforms to the conditions specified in bit 0-2."

When the sw flag is 1, the X and Y inputs are swapped.

When the zx flag is 1, the left operand is replaced with 0.

## 3. Control Unit

"In addition to the ALU instructions, the computer should also support data instructions. In a data instruction, the instruction value is directly written to the A register.

Create a control unit which execute either a data- or ALU instruction, depending on the high-bit of the instruction I:"

"For a data instruction, the output R should be the I input, and destination should be the A register. I.e. a should be 1 and d, a*, and j flags should be 0"

## 4. Computer

"Build a computer by combining:

  - A control unit

  - Storage memory (RAM and registers)

  - A program memory unit (ROM).

  - A counter which keeps track of the current instruction address (called the "program counter" or PC).

  - A clock unit

The word at the PC address in the program memory is the I input to the control unit.

Each clock cycle changes the program counter depending on j:

If j=0, the program counter should advance with 1.

If j=1, the PC should be set to the value on A."

## 5. Input/Output

Simple input/output exercise using bits